NOTES:

SAMPLING

1.1 Sampling

Definition

Sampling is the process of converting an analog (continuous-time) signal into a discrete-time signal by taking measurements at equal intervals.

Key Terms

- Sampling Frequency (Fs): Number of samples taken per second (Hz).

- Sampling Period (Ts): Time between samples

Ts=1Fs

1.2 Nyquist Sampling Theorem

To perfectly reconstruct the original signal from samples:

Fs≥2Fmax

- Fmax: Highest frequency of the analog signal

- If violated → aliasing

1.3 Aliasing

Occurs when signal contains components above Fs/2. These frequencies “fold” into lower frequencies.

Prevention

- Use Anti-Aliasing Filter (low-pass filter)

- Increase sampling rate

1.4 Types of Sampling

- Ideal Sampling: Multiplication with impulse train; theoretical only

- Natural Sampling: Sample follows shape of signal over pulse width

- Flat-Top Sampling: Sample amplitude held constant (real sampling circuits)

QUANTIZATION

Quantization converts infinite amplitude values of sampled signals into finite levels.

Quantization Error

Difference between actual sample and quantized value.

Step Size (Δ)

Δ=RangeL

Smaller Δ → smaller quantization error.

Number of Levels

L=2n(n = bits)

Types of Quantization

- Uniform: Equal step size

- Non-Uniform: Smaller steps at low amplitude (µ-law, A-law)

1.6 Signal-to-Quantization Noise Ratio (SQNR)

SQNR=6.02n+1.76 dB

QUANTUM COMPUTING

2.1 Introduction

Quantum computers use principles of quantum mechanics to process information faster than classical computers for specific tasks.

2.2 Core Concepts

Qubits

Quantum version of bits. Can be:

- 0

- 1

- Superposition of both

Superposition

Allows qubits to represent many states simultaneously → exponential speedup.

Entanglement

Two or more qubits share correlated states; changing one affects the other instantly.

Interference

Used to amplify correct answers and cancel wrong ones.

Quantum Gates

Operations that change qubit states (Hadamard, Pauli-X, CNOT).

2.3 Applications

- Cryptography (Quantum-safe systems)

- Drug discovery & molecular simulation

- Optimization (logistics, scheduling)

- Machine Learning (quantum neural networks)

- Financial modeling

- Material science

📘 3. Edge Computing

3.1 Definition

Edge computing processes data close to the source (e.g., sensors) rather than sending everything to cloud servers.

3.2 Why Edge Computing?

- Lower latency

- Real-time processing

- Reduced bandwidth

- Data privacy and security

- Works with poor connectivity

3.3 Edge Architecture

- Edge Devices: IoT sensors, smartphones, cameras

- Edge Nodes: Local processors

- Fog Layer (optional): Mini data centers

- Cloud: Long-term storage and global orchestration

4. Comparison Tables

4.1 DSP Sampling vs Quantization

| Feature | Sampling | Quantization |
| --- | --- | --- |
| Domain | Time | Amplitude |
| Output | Discrete time | Discrete amplitude |
| Issues | Aliasing | Quantization noise |
| Key Parameter | Fs | Bits (n) |
| Helps in | Converting analog → discrete | Digital encoding |

4.2 Quantum vs Edge Computing

| Feature | Quantum Computing | Edge Computing |
| --- | --- | --- |
| Basis | Quantum mechanics | Distributed processing |
| Main Feature | Superposition + entanglement | Low latency/data locality |
| Use Cases | Optimization, cryptography | IoT, real-time systems |
| Hardware | Qubits, cryogenic systems | Sensors, gateways |
| Maturity | Early-stage | Mature & widely deployed |

4.3 DSP vs Edge Computing

| Feature | DSP | Edge Computing |
| --- | --- | --- |
| Purpose | Signal processing | Local computation |
| Domain | Mathematics-based | Network-based |
| Use Cases | Audio, speech, filtering | IoT, AI, automation |
| Inputs | Analog signals | Sensor + digital data |
| Overlap | Edge devices use DSP for preprocessing |  |

SUMMARY:

- Quantum computing is an advanced computing technology based on quantum mechanics, the science of atomic and subatomic particles.

- Unlike classical computers that use bits (0 or 1), quantum computers use qubits, which can represent 0, 1, or both at the same time due to superposition.

- This gives quantum computers the ability to perform parallel computations, making them extremely powerful for certain tasks.

- Another key property, entanglement, links two or more qubits so that the state of one instantly affects the other, enabling complex multi-qubit operations.

- Quantum operations are performed using quantum gates, which manipulate qubit states similar to logic gates in classical computers, but with more complex behavior.

- Quantum computing can solve some problems much faster than classical supercomputers, especially in optimization, simulation, and cryptography.

- It has the potential to revolutionize fields like:

- Cryptography (breaking and creating secure systems)

- Drug discovery & molecular simulation

- Artificial intelligence and machine learning

- Optimization problems in engineering and logistics

- Material science and climate modelling

- Edge computing is a modern computing model where data is processed close to the source (edge of the network), such as IoT devices or local servers.

- Instead of sending all data to a distant cloud, edge computing performs computation locally, reducing the distance data must travel.

- This approach dramatically improves:

- Speed (low latency)

- Real-time responsiveness

- Network efficiency (less bandwidth usage)

- Traditional cloud computing can cause problems like high delay, network congestion, privacy risks, and dependence on internet connectivity.

- Edge computing solves these issues by handling most processing at the local node and sending only necessary information to the cloud.

Edge computing is essential for modern technologies like:

- IoT (Internet of Things)

- 5G networks

- Real-time analytics

- AI at the edge (smart cameras, wearables, sensors)

- Autonomous vehicles

- Smart cities (traffic, energy systems)

- Industrial automation (Industry 4.0)

- Healthcare (real-time monitoring)

- Agriculture (smart farming)